DATASET REPORT: POLAR GRID

Project: Polar Grid - AI-Assisted Renewable Energy Forecasting & Microgrid Optimization

Target Station: Mawson Station, Antarctica (67.6027° S, 62.8738° E)

Date: September 2026

Status: MVP Phase 1 Inspection Complete

_________________________________________________________________________________

1. Summary of Available Datasets

| Dataset File / Source | File Type | Time Period | Native Resolution | Key Useful Columns | Limitations |
| --- | --- | --- | --- | --- | --- |
| `SOE_SFU.zip` (indicator_59.csv) | CSV | 1986 – 2016 (30 years) | Monthly | Date, Place='Mawson', Parameter='Station electricity usage', Value (kWh) | Monthly aggregate only; no hourly station load data. |
| `SOE_SFU.zip` (indicator_56.csv) | CSV | 1993 – 2016 (23 years) | Monthly | Date, Place='Mawson', Parameter='Fuel usage', Value (Litres) | Monthly aggregate of diesel fuel consumed by generators & boilers. |
| `SOE_SFU.zip` (indicator_58.csv) | CSV | 1993 – 2016 | Monthly | Date, Place, Value (Litres) | Vehicle fuel only (not stationary power generation). |
| `SOE_SFU.zip` (indicator_57.csv) | CSV | 1995 – 2016 | Monthly | Date, Place, Value (Litres) | Incinerator fuel only. |
| `3491775ab61d2e083fb4ef37e09ee91a.grib` | Binary GRIB1 (5.5 KB) | Year 2010 | Monthly (12 monthly slices) | param_165 (10m U wind), param_166 (10m V wind), param_167 (2m temperature), param_169 (surface solar radiation downwards) | Single point for Mawson, only 1 snapshot per month. Insufficient by itself for 24–72h dynamic operational dispatch. |
| `climate_temps.zip` | TXT files | Historical | Monthly | Mean air temperatures for sub-Antarctic islands | Excludes hourly Antarctic continental station data. |
| `feart-10-961799.pdf` | PDF | 1986 – 2020 | Publication | Solar radiation reconstruction and ERA5 comparison literature | Background reference paper. |
| ERA5 Hourly Reanalysis (ECMWF via Open-Meteo Archive) | Reanalysis API / CSV | Continuous | Hourly | temperature_2m, wind_speed_10m, wind_direction_10m, shortwave_radiation, direct_normal_irradiance, diffuse_radiation | Open reanalysis model data; required to enable genuine 24–72 hour operational forecasting. |

_________________________________________________________________________________

2. Detailed Dataset Inspections

2.1 Station Electricity Usage (indicator_59.csv)

• Source: Australian Antarctic Data Centre (AADC), Commonwealth of Australia.

• Total Records for Mawson: 360 monthly records (Jan 1986 – Feb 2016).

• Missing Values: 8 missing months out of 360 (97.8% data completeness).

• Typical Range for Mawson:

⁃ Monthly usage: 110,000 to 175,000 kWh/month.

⁃ Equivalent continuous average electrical load: ~150 kW to ~240 kW, peaking during Antarctic winter (June–August) due to increased heating, lighting, and indoor activity.

2.2 Station Generator Fuel Usage (indicator_56.csv)

• Source: AADC, Commonwealth of Australia.

• Total Records for Mawson: 278 monthly records (Jan 1993 – Feb 2016).

• Missing Values: 0 missing records in the recorded range.

• Typical Range for Mawson:

⁃ Monthly diesel fuel usage: 44,000 to 75,000 litres/month.

⁃ Specific fuel consumption empirical ratio: ~0.30 to ~0.38 litres/kWh, perfectly consistent with heavy-duty polar diesel generator operations.

2.3 ERA5 Weather Data (3491775ab61d2e083fb4ef37e09ee91a.grib & Hourly ERA5)

• Coordinates: Mawson Station (-67.6027° S, 62.8738° E).

• Variables:

⁃ t2m (2m Temperature): -25°C to +2°C.

⁃ wind_speed_10m: 5 m/s to >30 m/s (Mawson is renowned as one of the windiest coastal stations in Antarctica).

⁃ solar_radiation: 0 W/m² during polar night (May–July), reaching up to 600–800 W/m² during polar day (December–January).

_________________________________________________________________________________

3. Transparency & Data Integrity Classifications

As mandated by the project integrity guidelines:

• REAL HISTORICAL DATA:

⁃ Mawson monthly electricity consumption (1986–2016).

⁃ Mawson monthly generator diesel fuel consumption (1993–2016).

⁃ Monthly ERA5 reanalysis single-level data (2010).

• REANALYSIS DATA:

⁃ Hourly ECMWF ERA5 weather parameters (wind speed, solar irradiance, ambient temperature) for Mawson Station coordinates.

• MODELED DATA:

⁃ Hourly load profile synthesized using physical thermal degree-hour formulas and diurnal human activity profiles, strictly calibrated so that the monthly integral matches the actual historical monthly consumption in indicator_59.csv.

⁃ Solar PV generation modeled from solar irradiance, ambient temperature, and panel efficiency.

⁃ Wind generation modeled from wind speed and turbine power curves.

• FORECASTED DATA:

⁃ Machine learning multi-step predictions for temperature, wind, solar irradiance, and demand using chronological splits.

• OPTIMIZED DISPATCH:

⁃ Recommended hourly schedule computed via Linear Programming (LP/MILP) minimizing diesel fuel while respecting battery SoC and power balance constraints.

_________________________________________________________________________________

4. Preprocessing Pipeline Design

1. Extraction & Isolation:

⁃ Extract CSVs from SOE_SFU.zip into data/raw/ without altering the originals in DataSet/.

2. Cleaning & Standardization:

⁃ Filter Place == 'Mawson'.

⁃ Parse Date (e.g. 'Jan-86') into standardized datetime format.

⁃ Clean numerical values, handle NaNs via forward/linear interpolation.

⁃ Rename to canonical column names: timestamp, electricity_kwh, fuel_litres.

3. Hourly Alignment:

⁃ Combine with hourly ERA5 meteorological time series.

⁃ Synthesize and calibrate the hourly demand profile to real monthly aggregates.

⁃ Save the unified, clean dataset to data/processed/mawson_hourly_energy_weather.csv.