POLAR GRID: 1-PAGE TESTING & VALIDATION SUMMARY

Project: Polar Grid – AI-Assisted Renewable Energy Forecasting & Microgrid Dispatch Optimization

Auditor: Antigravity Autonomous Systems Validation Engine

Date: September 2026

_________________________________________________________________________________

🏆 MVP STATUS VERDICT

READY WITH WARNINGS

The MVP pipeline, optimization engine, FastAPI backend, and React dashboard run without errors. The warnings are scientific communication guidelines for the judges (not code crashes).

_________________________________________________________________________________

📊 Quick Validation Scorecard

| Area | Tests Run | Result | Key Note |
| --- | --- | --- | --- |
| 1. Raw Datasets | 3 | PASS | Original files in DataSet/ remain 100% untouched. 360 monthly records extracted. |
| 2. Preprocessing & Load | 5 | PASS | 8,760 hourly records generated. Calibrated demand matches real monthly average within 0.65%. |
| 3. Data Leakage | 4 | PASS | Chronological 80/20 train/test split. Lags and rolling means strictly use $t-1$ to $t-24$. |
| 4. ML Forecasting | 4 | PASS | HistGradientBoosting achieves $R^2 = 0.966 - 0.992$. Finite, non-negative predictions. |
| 5. Solar PV Model | 5 | PASS | Zero power at night. Sub-zero temperature boost works. Capped at 100 kW rating. |
| 6. Wind Turbine Model | 11 | PASS | Cut-in at 3.5 m/s, rated at 12 m/s, storm shutdown at 25 m/s. Capped at 200 kW rating. |
| 7. Battery (BESS) | 3 | PASS | SoC strictly bounded between 20% and 95%. 90% round-trip efficiency enforced. |
| 8. Diesel Generator | 2 | PASS | Capacity capped at 375 kW. Fuel curve accurately follows $0.24 P_{\text{gen}} + 0.04 P_{\text{rated}}$. |
| 9. Microgrid Optimization | 3 | PASS | Power balance preserved to machine precision ($2.84 \times 10^{-14}\text{ kW}$). HiGHS LP solver. |
| 10. Baseline Audit | 2 | PASS / WARN | 78.57% reduction verified for late-December summer. Winter reduction drops to 26.19%. |
| 11. Multi-Horizon | 3 | PASS | 24h (100% reduction), 48h (78.9% reduction), and 72h (66.2% reduction) execute cleanly. |
| 12. API & Frontend | 13 | PASS | All FastAPI endpoints return 200 OK. Interactive React dashboard verified in browser. |
| 13. Stress & Edge Cases | 14 | PASS | Tested storms, calm air, polar night, demand spikes, empty/full battery, and missing data. |
| 14. End-to-End Pipeline | 1 | PASS | python run_pipeline.py executes in 8.36 seconds. |

_________________________________________________________________________________

🔍 The Two Critical Audit Findings (Explain to Judges)

1. Why are the ML $R^2$ scores so high ($0.966$ to $0.992$)?

• Scientific Reason: The model performs a 1-step ahead rolling forecast with true state persistence (predicting hour $t$ using known $t-1$ observations).

• Meteorological variables (temperature and ambient weather) exhibit strong physical autocorrelation over 1 hour.

• Diurnal solar and heating curves are tightly captured by $\sin/\cos(\text{hour})$ and day-of-year features.

• What to tell judges: "Our model uses short-term persistence and cyclical temporal encoding for real-time dispatch, updating predictions hourly as station SCADA measurements arrive."

2. Is the 78.57% diesel reduction real, and will it happen year-round?

• Scientific Reason: The 48-hour prototype test occurs in late December (Austral Summer).

• In December, Antarctica has 24-hour sunlight and Mawson Station experiences strong $10.2\text{ m/s}$ average winds.

• Clean renewable generation potential ($7,669\text{ kWh}$) actually exceeded station demand ($7,364\text{ kWh}$)!

• In Polar Night (July), solar generation is $0\text{ kWh}$, and diesel reduction drops to $26.19\%$.

• What to tell judges: "78.57% is the simulated dispatch reduction during peak polar summer; in polar winter, the system achieves ~26% reduction via wind, yielding a projected annual average saving of ~38%–45% (~250,000 litres of diesel saved per year)."

_________________________________________________________________________________

🚀 Top 5 Things to Do Before the SIH Presentation

1. Clearly State Data Provenance: State upright that historical station consumption was available as monthly totals and was transparently calibrated to an hourly thermodynamic heating profile.

2. Present Summer vs. Winter Contrast: Show judges both the December summer demo (~78% saving) and the July polar night demo (~26% saving). This proves scientific integrity and will impress judges.

3. Highlight Antarctic Logistics Economics: Translate litres saved into dollars. Diesel in Antarctica costs $3 to $7 USD per litre to transport via icebreaker. Saving 1,950 L in 48h saves $6,000–$14,000 USD and prevents ~5.2 tonnes of $\text{CO}_2$ emissions.

4. Demonstrate Dynamic Horizon Switching: In the dashboard, click the "24 Hours", "48 Hours", and "72 Hours" buttons live to show the linear programming solver re-calculating the schedule instantly.

5. Point to the Clean Codebase: Mention that unit tests pass in 0.098 seconds and the entire end-to-end pipeline runs in under 9 seconds via python run_pipeline.py.