# Module 03: Weather & Live Forecast

## 1. What is this module?

This module connects Polar Grid to live numerical weather prediction services. It ingests hourly weather forecasts for Mawson Station directly from the European Centre for Medium-Range Weather Forecasts (ECMWF IFS) and supplies the physical environmental inputs needed to estimate future solar generation, wind power, and station heating demand.

## 2. Why do we need it?

You cannot operate a microgrid reliably by looking only at yesterday's weather. Solar irradiance depends on cloud cover and solar zenith angles, while wind turbine power is proportional to the cube of wind velocity ($v^3$). In Antarctica, a sudden blizzard or calm can occur within 2 hours. By obtaining a high-resolution forward weather forecast, Polar Grid can anticipate renewable generation 24 to 72 hours ahead and schedule battery reserves before diesel is needed.

## 3. What Polar Grid actually does

1. Live Weather Ingestion: Calls the Open-Meteo ECMWF API endpoint requesting 6 essential meteorological parameters at Mawson's coordinates (-67.6027° S, 62.8738° E):

- temperature_2m (°C)

- wind_speed_10m (m/s, queried with &wind_speed_unit=ms)

- wind_direction_10m (degrees)

- shortwave_radiation (Global Horizontal Irradiance, W/m²)

- direct_normal_irradiance (W/m²)

- diffuse_radiation (W/m²)

2. Local Caching: Persists successful live forecasts to data/processed/latest_ecmwf_forecast.json to prevent API rate limits and provide resilient offline demonstrations.

3. Graceful Fallback Mechanism:

- If the external API is unreachable (HTTP 503 or network timeout), the service automatically falls back to the cached forecast.

- If no cache exists, it falls back to a clean historical ERA5 reanalysis slice for that season.

- The system strictly declares its status in the API metadata (mode: 'live', 'cached', or 'fallback') so the dashboard never falsely claims live data if cached data is used.

4. Physical Decoupling:

- ECMWF provides the weather forecast.

- Polar Grid converts that weather forecast into renewable kilowatts.

- The ML demand model estimates station electricity load based on the forecast temperature.

- The HiGHS optimizer uses both to schedule battery and diesel dispatch.

+-----------------------------------------------------------------------------------+

|                        WEATHER INTEGRATION ARCHITECTURE                           |

+-----------------------------------------------------------------------------------+

|                         [ ECMWF IFS Supercomputer ]                               |

|                                     |                                             |

|                                     v                                             |

|                     [ Open-Meteo REST API Gateway ]                               |

|                                     |                                             |

|             +-----------------------+-----------------------+                     |

|             | (Online)                                      | (Offline/Timeout)   |

|             v                                               v                     |

|    [ Live ECMWF Forecast ]                       [ Local Cache / ERA5 ]           |

|  (24h/48h/72h Hourly Data)                      (Transparent Fallback)            |

|             |                                               |                     |

|             +-----------------------+-----------------------+                     |

|                                     v                                             |

|                        [ WeatherForecastService ]                                 |

|                                     |                                             |

|             +-----------------------+-----------------------+                     |

|             |                                               |                     |

|             v                                               v                     |

|  [ Solar & Wind Physics ]                     [ Thermal Demand Model ]            |

|   PV & Turbine Kilowatts                       Station Electrical Load            |

+-----------------------------------------------------------------------------------+

## 4. Input

API Endpoint: https://api.open-meteo.com/v1/ecmwf

Location: Latitude -67.6027, Longitude 62.8738 (Mawson Station).

Forecast Horizons: 24, 48, or 72 hours.

## 5. Processing

Validates that returned values are non-empty and non-negative (wind $\ge 0$, solar $\ge 0$).

Rounds values to standard scientific precision (2 decimal places).

Extracts direct and diffuse components for solar cell angle adjustments.

## 6. Output

A standardized dictionary and pandas DataFrame containing hourly weather variables, metadata timestamps, and operational flags (is_live: True/False, source: "ECMWF IFS").

## 7. Simple Example

ECMWF Live Query: Ingests forecast for tomorrow at 14:00 UTC:

- Temperature: $-16.5^\circ\text{C}$

- Wind Speed: $14.2 \text{ m/s}$

- Solar Irradiance: $320 \text{ W/m}^2$

Downstream Translation:

- Demand model predicts station heating load will rise to 175 kW due to $-16.5^\circ\text{C}$ cold.

- Wind estimator predicts 200 kW (both turbines at full rated speed).

- Optimizer plans to charge the battery with the surplus 25 kW.

## 8. What should I say to a judge?

*"Judges, a critical principle of Polar Grid is that we do NOT try to predict tomorrow's weather with a simple ML model. Global weather is governed by complex atmospheric fluid dynamics handled by supercomputers. We ingest live numerical weather forecasts from the European Centre for Medium-Range Weather Forecasts (ECMWF IFS)—the world's most accurate global weather model. Polar Grid takes that high-quality weather forecast and converts it into physical kilowatt generation and thermal station demand to drive dispatch optimization."*

## 9. Common Judge Questions

1. Q: Is your weather forecast really live?

*A: Yes. When connected to the internet, Polar Grid queries Open-Meteo's ECMWF endpoint in real time for Mawson Station. If offline, it falls back to local cache or ERA5 and transparently labels the status.*

2. Q: Why don't you train an ML model to predict tomorrow's wind and solar yourself?

*A: Training a small local ML model on single-station historical data to predict multi-day weather ignores global pressure systems, jet streams, and Antarctic polar vortex dynamics. Professional meteorological institutes like ECMWF run petabyte-scale numerical weather prediction models on supercomputers. Using ECMWF is the industry standard for microgrid operators.*

3. Q: How accurate is ECMWF in Antarctica?

*A: ECMWF IFS is recognized by the World Meteorological Organization as the top-performing global model for polar latitudes, assimilating satellite soundings, Antarctic weather balloon data, and coastal automatic weather stations.*

4. Q: What happens if your internet connection drops in Antarctica?

*A: Polar Grid features an automatic fallback mechanism: it uses the latest cached forecast, and if completely offline, defaults to high-resolution ERA5 historical reanalysis data while clearly alerting the station operator.*

5. Q: What is the difference between ERA5 and ECMWF IFS?

*A: ERA5 is ECMWF's historical reanalysis (past recorded observations used for our ML training and backtesting). ECMWF IFS is their operational forecasting system (future predictions used for live dispatch).*

## 10. Important Things NOT to Claim

DO NOT claim Polar Grid invented its own weather forecasting physics engine (we use ECMWF IFS).

DO NOT claim the system predicts cloud formations or wind gusts from scratch with a small neural network.

DO NOT claim the live weather feed is 100% infallible (weather forecasts inherently have uncertainty, which is why battery buffers and diesel backup are retained).